- Feature relevance and identifiability analysis

A comprehensive feature relevance and identifiability analysis was conducted for the four normalized datasets considered in this study: relative frequency shifts (RFS), damage location coefficients (DLC), damaged natural frequencies (fdamaged​), and frequency changes (Δf). For each dataset, the features correspond to the modal quantities associated with the first ten vibration modes, and the target variable is the crack position along the beam. All analyses were performed exclusively on the training split to avoid information leakage.

Feature importance was evaluated using a set of complementary criteria capturing distinct aspects of relevance: monotonic physical trends (Spearman correlation), general statistical dependence (mutual information), nonlinear predictive reliance (random-forest permutation importance), and identifiability (single-feature cross-validated R2). These criteria were combined into a consensus ranking to identify features that are consistently informative across multiple perspectives.

- Relative Frequency Shift (RFS)

For the RFS dataset, the results indicate a clear dominance of the lowest vibration modes. Mode 1 exhibits the strongest relevance across all metrics, with a high absolute Spearman correlation (ρs​≈0.83), the largest mutual information, the largest permutation-importance score, and the highest single-feature cross-validated R2 (≈0.34). This implies that the RFS associated with the first mode alone explains a substantial fraction of the spatial variability in crack position.

Modes 2 through 5 show progressively decreasing relevance, but remain consistently ranked above the higher modes in the consensus ordering. In contrast, modes above the fifth exhibit very weak individual identifiability, with near-zero or negative single-feature R2, and negligible permutation importance. This behavior reflects the increasing spatial oscillation of higher modes, which reduces their ability to uniquely localize damage when considered in isolation.

Overall, the RFS representation concentrates most of the usable localization information in the first few modes, suggesting a strongly redundant feature structure dominated by low-order modal responses.

- Damage Location Coefficients (DLC)

The DLC dataset exhibits a markedly different relevance structure. As expected, DLC mode 1 overwhelmingly dominates all criteria, with an almost perfect monotonic relationship with crack position (ρs​≈0.99) and a very high single-feature R2 (≈0.89). This confirms that the normalized curvature-based formulation embedded in the DLC strongly emphasizes spatial localization effects for the first mode.

Unlike the RFS case, however, the subsequent ranking is non-monotonic with respect to mode number. While mode 2 remains important, several higher modes (notably modes 9 and 10) appear among the top five features according to the consensus ranking. These modes exhibit relatively weak linear or monotonic correlations but retain non-negligible mutual information and permutation importance, indicating that they encode complementary, nonlinear localization cues once the dominant contribution of mode 1 is accounted for.

This result highlights a key distinction of the DLC representation: by normalizing modal curvatures at each spatial location, the DLC suppresses magnitude effects and redistributes information across modes, enabling higher-order modes to contribute meaningfully to crack localization despite their weak standalone trends.

- Damaged natural frequencies

The feature relevance patterns for the damaged-frequency dataset closely mirror those observed for the RFS dataset. The first mode again emerges as the most informative feature across all metrics, followed by a monotonic decrease in relevance from modes 2 to 5. Higher modes provide little additional identifiability, as reflected by near-zero or negative single-feature R2 values and low permutation importance.

This similarity is expected, given that the damaged frequencies are derived directly from the healthy frequencies scaled by (1-RFS). As a result, the damaged-frequency representation largely preserves the information structure of the RFS while embedding it in absolute frequency values.

- Frequency change (Δf)

The Δf dataset yields feature relevance patterns that are almost identical to those of the RFS dataset. The same ordering of modes is obtained, with modes 1 through 5 forming the top tier of the consensus ranking and higher modes contributing marginally. Quantitatively, the relevance metrics for Δf are nearly indistinguishable from those of RFS, which is consistent with the fact that Δf is proportional to the RFS scaled by the healthy frequencies.

From a modeling perspective, this indicates that Δf and RFS encode essentially the same localization information, differing primarily in numerical scaling rather than in information content.

- Cross-representation comparison and implications

Taken together, these results reveal two distinct feature-information regimes:

- Magnitude-based representations (RFS, fdamaged​, Δf)
In these datasets, crack localization information is heavily concentrated in the lowest vibration modes, with rapidly diminishing returns from higher modes. Feature identifiability saturates early, indicating substantial redundancy and suggesting that compact or sparsely connected models may be sufficient.

- Normalized, relative representation (DLC)
The DLC dataset exhibits a more distributed relevance pattern, where higher-order modes retain nontrivial importance despite weak linear trends. This indicates that localization information is encoded through relative modal dominance rather than absolute magnitude, favoring models capable of capturing nonlinear interactions between modes.

These findings provide a clear physical and statistical justification for exploring different neural-network architectures in subsequent stages of the study. In particular, the strong redundancy observed in RFS- and Δf-based features motivates sparse or low-connectivity architectures, while the distributed relevance structure of the DLC features suggests potential benefits from nonlinear, modular, or small-world network topologies.